DatexDataPacket ::= SEQUENCE

{

datex-Version-number	ENUMERATED

{

experimental (0),

version1 (1),

...

},

datex-Version-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Version_code-datex15"

CONTEXT "ITS"

DEFINITION "This is the DATEX-ASN version to which this message conforms"

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Version"

DATA TYPE ENUMERATED { experimental, version-1, …}

VALUE RULE "ENUMERATED { experimental, version-1, …}"

}

datex-Data		OCTET STRING,

datex-Data-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Data_text-general"

CONTEXT "ITS"

DEFINITION "The optionally encrypted contents of the DATEX Data Packet as defined by the structure C2CAuthenticatedMessage."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-OctetString Unlimited"

DATA TYPE OCTET STRING

VALUE RULE “OCTET STRING”

CONSTRAINTS "The initial layout for this octet string is given by C2CAuthenticatedMessage;

however, this data may be encrypted prior to transmission over the communications media."

}

datex-Crc-nbr	OCTET STRING (SIZE (2))

datex-Crc-id DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Crc_id-crc16"

CONTEXT "ITS"

DEFINITION "The reletivaly unique code for the message byte stream which is used for error checking.

The value of the code is determined by applying the CRC16 algorithm defined in ISO 3309 on the byte stream encoding of datex-Data-txt. For example, if BER encoding was used, the CRC would be calculated on the identifier, length, and contents octets of the encoding."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

KEYWORDS "CRC, DATEX"

VALUE DOMAIN "Id-Crc16"

DATA TYPE OCTET STRING (SIZE (2))

VALUE RULE "OCTET STRING (SIZE (2))"

}

}

C2CAuthenticatedMessage ::= SEQUENCE {

datex-AuthenticationInfo-text	OCTET STRING (SIZE (0..255)),

datex-AuthenticationInfo-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_AuthenticationInformation_text-general255"

CONTEXT "ITS"

DEFINITION "Authentication information which the two interconnected systems have agreed to include within each message.

This can be used to exchange a fixed code or algorithmic code in order to provide additional password protection. This may be zero length octet string, a fixed octet string, or a variable octet string according to rules defined in the Interchange Agreement (e.g., the parameters defined in Clause 5)."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

KEYWORDS "Authentication, DATEX"

VALUE DOMAIN "Text-OctetString 255"

DATA TYPE OCTET STRING (SIZE (0..255))

VALUE RULE "OCTET STRING (SIZE (0..255))"

}

datex-DataPacket-number	INTEGER (0..4294967295),

datex-DataPacket-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_DataPacket_number-ulong"

CONTEXT "ITS"

DEFINITION "Indicates the data packet number for the data packet being sent.

The first data packet sent during session establishment shall have a number of zero. Each subsequent data packet shall have its number incremented by one. The client and server shall maintain separate counters such that the first data packet sent by the server and the first data packet sent by the client shall both be zero."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

datex-DataPacketPriority-number	INTEGER (0..10),

datex-DataPacketPriority-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_DataPacketPriority_code-datex11"

CONTEXT "ITS"

DEFINITION "The priority of the message. A system must process messages in the order in which they are received,

unless a later message has a higher priority. Messages of higher priority may be processed before messages of lower priority at the option of the implementation."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Priority"

DATA TYPE INTEGER (1..10)

VALUE RULE " INTEGER (1..10)"

}

options			HeaderOptions,

pdu			PDUs

}

HeaderOptions ::= SEQUENCE

{

datex-Origin-text		UTF8String (SIZE (0..40))	OPTIONAL,

datex-Origin-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Origin_text-name"

CONTEXT "ITS"

DEFINITION "The domain name of the system that collected the data contained in the end application message."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-Name"

DATA TYPE UTF8String (SIZE (0..40))

VALUE RULE "UTF8String (SIZE (0..40))"

}

datex-OriginAddress-location	OCTET STRING		OPTIONAL,

datex-OriginAddress-loc DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_OriginAddress_location-address"

CONTEXT "ITS"

DEFINITION "A unique address of the system that collected the data contained in the end-application."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Location-Address"

DATA TYPE OCTET STRING

VALUE RULE "AddressCode ::= CHOICE

{

gis IMPLICIT [1] NumericString,

mhORName IMPLICIT [2] MhORName, --X.400

dn IMPLICIT [3] DistinguishedName, -- X.500

isdnOrPhonenumber IMPLICIT [4] E164Form,

rfc822Address IMPLICIT [5] PrintableString,

pstnAddress IMPLICIT [6] NumericString

}"

}

datex-Sender-text		UTF8String (SIZE (0..40))	OPTIONAL,

datex-Sender-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Sender_text-name"

CONTEXT "ITS"

DEFINITION "The domain name of the system which is sending the message."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-Name"

DATA TYPE UTF8String (SIZE (0..40))

VALUE RULE "UTF8String (SIZE (0..40))"

}

datex-SenderAddress-location	OCTET STRING		OPTIONAL,

datex-SenderAddress-loc DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_SenderAddress_location-address"

CONTEXT "ITS"

DEFINITION "A unique address for the computer which is sending the message."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Location-Address"

DATA TYPE OCTET STRING

VALUE RULE "AddressCode ::= CHOICE

{

gis IMPLICIT [1] NumericString,

mhORName IMPLICIT [2] MhORName, --X.400

dn IMPLICIT [3] DistinguishedName, -- X.500

isdnOrPhonenumber IMPLICIT [4] E164Form,

rfc822Address IMPLICIT [5] PrintableString,

pstnAddress IMPLICIT [6] NumericString

}"

}

datex-Destination-text		UTF8String (SIZE (0..40))	OPTIONAL,

datex-Destination-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_Destination_text-name"

CONTEXT "ITS"

DEFINITION "The domain name of the system which is supposed to receive the message."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

KEYWORDS "DATEX, Destination"

VALUE DOMAIN "Text-Name"

DATA TYPE UTF8String (SIZE (0..40))

VALUE RULE "UTF8String (SIZE (0..40))"

}

datex-DestinationAddress-location	OCTET STRING		OPTIONAL,

datex-DestinationAddress-loc DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_DestinationAddress_location-address"

CONTEXT "ITS"

DEFINITION "A unique address for the computer which is the intended recipient of the message."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Location-Address"

DATA TYPE OCTET STRING

VALUE RULE "AddressCode ::= CHOICE

{

gis IMPLICIT [1] NumericString,

mhORName IMPLICIT [2] MhORName, --X.400

dn IMPLICIT [3] DistinguishedName, -- X.500

isdnOrPhonenumber IMPLICIT [4] E164Form,

rfc822Address IMPLICIT [5] PrintableString,

pstnAddress IMPLICIT [6] NumericString

}"

}

datex-Cost			Cost			OPTIONAL,

datex-DataPacket-time		IsoTime			OPTIONAL

datex-DataPacketTime DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.MESSAGE_DataPacketTime_frame"

CONTEXT "ITS"

DEFINITION "The time at which the data packet is being generated."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

}

Cost ::= SEQUENCE

{

amount-Currency-code	 	OCTET STRING (SIZE (3)),

amount-Currency-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "AMOUNT_Currency_code-datex1"

CONTEXT "ITS"

DEFINITION "The three letter currency code associated with the quantity."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-Currency"

DATA TYPE OCTET STRING (SIZE (3))

VALUE RULE "ISO 4217"

}

amount-Factor-quantity		INTEGER,

amount-Factor-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "AMOUNT_Factor_quantity"CONTEXT "ITS"

DEFINITION "The 10^x factor applied to the value given in AMOUNT_Quantityquantity.

For example, if the Curency is USD, the Factor is -3, and the Quatity is 11;

the amount being specified would be US$0.011 or 1.1 US cents."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-unlimited"

DATA TYPE INTEGER

VALUE RULE “INTEGER”

}

amount-Quantity-quantity	INTEGER

amount-Quantity-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "AMOUNT_Quantity_quantity"

CONTEXT "ITS"

DEFINITION "The quantity of units as specified by currency and factor."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-unlimited"

DATA TYPE INTEGER

VALUE RULE “INTEGER”

}

}

PDUs

Initiate ::= SEQUENCE {

datex-Sender-txt UTF8String (0..40),

datex-Destination-txt UTF8String (0..40)

}

Login ::= SEQUENCE

{

datex-Sender-txt			UTF8String (SIZE (0..40)),

datex-Destination-txt			UTF8String (SIZE (0..40)),

datexLogin-UserName-txt			OCTET STRING,

datexLogin-UserName-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_UserName_text-general"

CONTEXT "ITS"

DEFINITION "An optionally encrypted user-name for the Login request. The encryption algorithm is not specified in this standard."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-OctetString Unlimited"

DATA TYPE OCTET STRING

VALUE RULE ”OCTET STRING”

}

datexLogin-Password-txt			OCTET STRING,

datexLogin-Password-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_Password_text-general"

CONTEXT "ITS"

DEFINITION "An optionally encrypted password for the login request.

The password and user name shall be verified by the end application to ensure that the user has appropriate rights.

The encryption algorithm is not specified in this standard."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-OctetString Unlimited"

DATA TYPE OCTET STRING

VALUE RULE ”OCTET STRING”

}

datexLogin-EncodingRules-id		SEQUENCE OF OBJECT IDENTIFIER,

datexLogin-EncodingRules-id DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_EncodingRules_id-oids"

CONTEXT "ITS"

DEFINITION "A listing of the encoding rules that the Client supports."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Id-Object Identifiers"

DATA TYPE SEQUENCE OF OBJECT IDENTIFIER

VALUE RULE "SEQUENCE OF OBJECT IDENTIFIER"

}

datexLogin-HeartbeatDurationMax-qty		INTEGER (0..65535),

datexLogin-HeartbeatDurationMax-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_HeartbeatDurationMax_quantity-ushort"

CONTEXT "ITS"

DEFINITION "The maximum duration, in seconds, allowed during the session without the exchange of data packets.

If this time is exceeded without any data packets being received from the other system,

the session shall be locally terminated without exchanging any data packets.

The value zero shall indicate that heartbeats are not being used."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-USHORT"

DATA TYPE INTEGER (0..65535)

VALUE RULE "INTEGER (0..65535)"

}

datexLogin-ResponseTimeOut-qty		INTEGER (0..255),

datexLogin-ResponseTimeOut-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_ResponseTimeOut_quantity-ubyte"

CONTEXT "ITS"

DEFINITION "The time in seconds within which a system will expect to receive a response to the transmission of a data packet.

This same timer will apply to both the Client and the Server and will apply to all data packets transmitted within the session where a response is required. The time is measured from the return from the system write () call. If a system has not received the appropriate response prior to this timer expiring, it will assume the transmitted data packet was not received by the other end and proceed as defined in Clause 6.1.4(Retransmit)."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-UBYTE"

DATA TYPE INTEGER (0..255)

VALUE RULE "INTEGER (0..255)"

CONSTRAINTS "The value zero (0) is not allowed."

}

datexLogin-Initiator-cd			ENUMERATED

{

serverInitiated,

clientInitiated,

...

},

datexLogin-Initiator-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_Initiator_code-datex2"

CONTEXT "ITS"

DEFINITION "Indicates who initiated the Login request."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Initiator"

DATA TYPE ENUMERATED {serverInitiated, clientInitiated, …}

VALUE RULE "ENUMERATED {serverInitiated, clientInitiated, …}"

}

datexLogin-DatagramSize-qty		INTEGER (0..65535)

datexLogin-DatagramSize-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGIN_DatagramSize_quantity-ushort"

CONTEXT "ITS"

DEFINITION "The maximum datagram size that will be supported during the session."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-USHORT"

DATA TYPE INTEGER (0..65535)

VALUE RULE “INTEGER (0..65535)”

}

}

FrED ::= INTEGER (0..4294967295)  -- datexFrED-ConfirmPacket-nbr

datexFrED-ConfirmPacket-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.FrED_ConfirmPacket_number-ulong"

CONTEXT "ITS"

DEFINITION "The data packet number being confirmed by the 'friendly exchange of data'data packet.

If the FrED is being used as a heartbeat, the value of this field shall be zero (0)."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

Terminate ::= ENUMERATED -- datexTerminate-Reason-cd

{

other,

serverRequested,

clientRequested,

serverShutdown,

clientShutdown,

serverCommProblems,

clientCommProblems,

...

}

datexTerminate-Reason-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.TERMINATE_Reason_code-datex14"

CONTEXT "ITS"

DEFINITION "Indicates who initiated the termination of the session and a reason."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Terminate"

DATA TYPE ENUMERATED {other, serverRequested, clientRequested, serverShutdown,

clientShutdown, serverCommProblems, clientCommProblems, …}

VALUE RULE "ENUMERATED {other, serverRequested, clientRequested, serverShutdown,

clientShutdown, serverCommProblems, clientCommProblems, …}"

}

Logout ::= ENUMERATED	-- datexLogout-Reason-cd

{

other,

serverRequested,

clientRequested,

serverShutdown,

clientShutdown,

serverCommProblems,

clientCommProblems,

...

}

datexLogout-Reason-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.LOGOUT_Reason_code-datex14"

CONTEXT "ITS"

DEFINITION "The reason the logout is occuring."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Terminate"

DATA TYPE ENUMERATED {other, serverRequested, clientRequested, serverShutdown,

clientShutdown, serverCommProblems, clientCommProblems, …}

VALUE RULE "ENUMERATED {other, serverRequested, clientRequested, serverShutdown,

clientShutdown, serverCommProblems, clientCommProblems, …}"

}

Subscription => ISO14827-Subscription.docx

IsoTime => Omit

Publication => ISO14827-Publication.docx

EndApplicationMessage ::= SEQUENCE

{

endApplication-Message-id	ISO14827-MESSAGE.&id,

endApplication-Message-id DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "END.APPLICATION_Message_id"

CONTEXT "ITS"

DEFINITION "The object identifier associated with the end application message that is contained within this data packet.

The definition of end application messages is outside the scope of this standard; however ISO 14827-1 defines the generic structure within which end application messages are defined."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "As defined by the End Application message"

DATA TYPE ISO14827-MESSAGE.&id

VALUE RULE "As defined by the End Application message"

}

endApplication-Message-msg	ISO14827-MESSAGE.&MessageBody

endApplication-Message-msg DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "END.APPLICATION_Message_msg"

CONTEXT "ITS"

DEFINITION "The encoded content of the end application message.

The definition of end application messages is outside the scope of this standard; however ISO 14827-1 defines the generic structure within which end application messages are defined."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "As defined by the End Application message"

DATA TYPE ISO14827-MESSAGE.&MessageBody

VALUE RULE "As defined by the End Application message"

}

}

TransferDone ::= SEQUENCE

{

--	datexTransferDone-Publication-nbr	INTEGER (0..4294967295), 		deprecated on ISO-14827-2:2005

datexTransferDone-FileName-txt	UTF8String (SIZE (0..2000)),

datexTransferDone-FileName-txt DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.TRANSFER.DONE_FileName_text-memo"

CONTEXT "ITS"

DEFINITION "The name of the file which the Client is no longer attempting to retrieve."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Text-Memo"

DATA TYPE UTF8String (SIZE (0..2000))

VALUE RULE "UTF8String (SIZE (0..2000))"

}

datexTransferDone-Success-bool	BOOLEAN

datexTransferDone-Success-bool DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.TRANSFER.DONE_Success_boolean"

CONTEXT "ITS"

DEFINITION "Indicates whether or not the file transfer was successful."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Boolean"

DATA TYPE BOOLEAN

VALUE RULE "BOOLEAN"

}

}

Accept ::= SEQUENCE

{

datexAccept-Packet-nbr		INTEGER (0..4294967295),

datexAccept-Packet-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.ACCEPT_Packet_number-ulong"

CONTEXT "ITS"

DEFINITION "The DATEX.MESSAGE_DataPacket_number-ulong, as defined in B.19, of the request which is being accepted."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

datexAccept-Type		CHOICE

{

login			OBJECT IDENTIFIER,	-- encoding rules

datexAccept-Login-id DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.ACCEPT_Login_id-oid"

CONTEXT "ITS"

DEFINITION "Indicates the OID of the encoding rules being accepted for the session."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Id-Object Identifier"

DATA TYPE OBJECT IDENTIFIER

VALUE RULE "OBJECT IDENTIFIER"

}

single-subscription 		NULL,

registered-subscription	INTEGER (0..4294967295),-- the accepted value for the UpdateDelay parameter

datexAccept-Registered-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.ACCEPT_Registered_number-ulong"

CONTEXT "ITS"

DEFINITION "Indicates the accepted value for the datexRegistered-UpdateDelay-qty

Parameter from the associated request."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

publication 		NULL

}

}

Reject ::= SEQUENCE

{

datexReject-Packet-nbr		INTEGER (0..4294967295),

datexReject-Packet-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_Packet_number-ulong"

CONTEXT "ITS"

DEFINITION "The DATEX.MESSAGE_DataPacket_number-ulong, as defined in A.1, of the request which is being rejected.

For the rejection of specific PublicationData structures within a Publication data packet, this number shall be set to zero."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

datexReject-Type		RejectType,			-- has to be checked

datexReject-AlternateRequest	AlternateRequest 	OPTIONAL

}

RejectType ::= CHOICE

{

datexReject-Login-cd		ENUMERATED

{

other,

unknownDomainName,

accessDenied,

invalidNamePassword,

timeoutTooSmall,

timeoutTooLarge,

heartbeatTooSmall,

heartbeatTooLarge,

sessionExists,

maxSessionsReached,

...

},

datexReject-Login-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_Login_code-datex6 "

CONTEXT "ITS"

DEFINITION "The reason that the Login packet is being rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Reject Login"

DATA TYPE ENUMERATED { other, unknownDomainName, accessDenied, invalidNamePassword, timeoutTooSmall, timeoutTooLarge,

heartbeatTooSmall, heartbeatTooLarge, sessionExists, maxSessionsReached, …}

VALUE RULE "other - used for any other reason

unknownDomainName - the Client or Server domain name in the Login was unknown or invalid

accessDenied - the Server is denying access for some reason

invalidNamePassword – the Server is denying access due to an invalid name password pair.

timeoutTooSmall (TooLarge) - the timeout value in the Login was not within a range that the Server supports

heartbeatTooSmall (TooLarge) - the heartbeat in the Login was not within a range that the Server supports

sessionExists - a session already exists between the indicated Client domain name and Server domain name over

the specified transport profile; only one session is allowed between the pair over the same profile.

maxSessionsReached - the Server can not support any more sessions."

}

datexReject-Subscription-cd	ENUMERATED

{

other,

unknownSubscriptionNbr,

invalidTimes,

frequencyTooSmall,

frequencyTooLarge,

invalid-mode,

publishFormatNotSupported,

unknowSubscriptionMsgId,

invalidSubscriptionMsgId,

invalidSubscriptionContent,

...

},

datexReject-Subscription-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_Subscription_code-datex8"

CONTEXT "ITS"

DEFINITION "A reason why the Subscription (request) was rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Reject Subscription"

DATA TYPE ENUMERATED { other, unknownSubscriptionNbr, invalidTimes, frequencyTooSmall,

frequencyTooLarge, invalidMode,

publishFormatNotSupported, unknownSubscriptionMsgId,

invalidSubscriptionMsgId, invalidSubscriptionContent, …}

VALUE RULE “other - used for any other error

unknownSubscriptionNbr - used if the subscription (request) is known to be too big to fit in a requested datagram

invalidTimes - used if the time of either the Request or event message is not recognized by the Server

frequencyTooSmall - used if the frequency is too small for one of the fields in the request or event message

within the subscription

frequencyTooLarge - used if the frequency is too large for one of the fields in the request or event message

within the subscription

invalidMode – the subscrption contained an invalid mode.

publishFormatNotSupported – the publish format requested within the subscription is not supported

unknownSubscriptionMsgId – the subscription message identification is unknown

invalidSubscriptionMsgId – the subscription message identification is invalid

invalidSubscriptionContent – the content of the subscription message is invalid”

}

datexReject-Publication-cd	ENUMERATED

{

other,

invalidPublishFormat,

...

},

datexReject-Publication-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_Publication_code-datex7"

CONTEXT "ITS"

DEFINITION "A reason why the Publication (reply) data packet was rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Reject Publication"

DATA TYPE ENUMERATED {other, invalidPublishFormat, …}

VALUE RULE " other - used for any other error

invalidPublishFormat – used if the selected publication format is invalid "

}

rejectPublicationData		SEQUENCE

{

datexReject-SubscriptionSerial-nbr	INTEGER (0..4294967295),

datexReject-SubscriptionSerial-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_SubscriptionSerial_number-ulong"

CONTEXT "ITS"

DEFINITION "The subscription serial number of the PublicationData structure that

contained invalid data and is being rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE " INTEGER (0..4294967295)"

}

datexReject-PublicationSerial-nbr	INTEGER (0..4294967295),

datexReject-PublicationSerial-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_PublicationSerial_number-ulong"

CONTEXT "ITS"

DEFINITION "The publication serial number of the PublicationData structure that

contained invalid data and is being rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE " INTEGER (0..4294967295)"

}

datexReject-PublicationData-cd	ENUMERATED

{

other,

unknownSubscription,

unknownPublicationNbr,

unknownPublicationMsgId,

invalidPublicationMsgId,

invalidPublicationMsgContent,

repeatedPublicationNbr,

...

}

datexReject-PublicationData-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REJECT_PublicationData_code-datex16"

CONTEXT "ITS"

DEFINITION "A reason why the PublicationData structure was rejected."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Reject Publication Data"

DATA TYPE ENUMERATED { other, unknownSubscription, unknownPublicationNbr,

unknownPublicationMsgId, invalidPublicationMsgId,

invalidPublicationMsgContent, repeatedPublicationNbr, …}

VALUE RULE " other - used for any other error

unknownSubscription - used if the subscription (request) is not recognized

unknownPublicationNbr - used if the publication is not recognized

unknownPublicationMsgId – used if the publication message identification number is not recognized

invalidPublicationMsgId - used if the publication message identification number is recognized,

but is an invalid message identification number

invalidPublicationMsgContent - the content of the publication message is recognized,

but the content is invalid

repeatedPublicationNbr – a publication with this publication number has already been received for

this subscription."

}

}

}