Subscription ::= SEQUENCE

{

datexSubscribe-Serial-nbr		INTEGER (0..4294967295),

datexSubscribe-Serial-nbr DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_Serial_number-ulong"

CONTEXT "ITS"

DEFINITION "The serial number for the subscription (request) being submitted.

Each subscription shall have an associated serial number; the number

may be used to update or cancel previously sent subscriptions . New

subscriptions shall not use a serial number which is already in use by

the Client/Server pair."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Number-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

CONSTRAINTS "The value of zero shall not be used"

}

datexSubscribe-Type			SubscriptionType,

...

}

SubscriptionType ::= CHOICE

{

subscription				SubscriptionData,

datexSubscribe-CancelReason-cd		ENUMERATED

{

other,

dataNotNeeded,

errorsInPublication,

pendingLogout,

processingMgmt,

bandwidthMgmt,

...

}

datexSubscribe-CancelReason-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_CancelReason_code-datex5"

CONTEXT "ITS"

DEFINITION "The reason that the subscription (request) is being cancelled. "

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Cancel Subscription"

DATA TYPE ENUMERATED {other, dataNotNeeded, errorsInPublication, pendingLogout,

processingMgmt, bandwidthMgmt, ...}

VALUE RULE "other - not one of the standard reasons listed in this standard

dataNotNeeded - indicates that the Client no longer requires the data

errorsInPublication - indicates that the Client is canceling the subscription (request) due to excessive publications that could not be decoded or contained invalid data.

pendingLogout - indicates that the Client is in the process of gracefully terminating the session

processingMgmt - indicates that the Client is cancelling the subscription so that it may concentrate its processing resources on other tasks

bandwidthMgmt - indicates that the Client is cancelling the subscription so that it may use the limited bandwidth for higher priority needs"

}

}

SubscriptionData ::= SEQUENCE

{

datexSubscribe-Persistent-bool		BOOLEAN,

datexSubscribe-Persistent-bool DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_Persistent_boolean"

CONTEXT "ITS"

DEFINITION " Indicates whether or not the subscription (request) will survive session terminations."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Boolean"

DATA TYPE BOOLEAN

VALUE RULE "BOOLEAN"

}

datexSubscribe-Status-cd		ENUMERATED { new, update },

datexSubscribe-Status-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_Status_code-datex12"

CONTEXT "ITS"

DEFINITION "An indication of the status of this subscription (request) message. If the value is 'new', the request number shall be unique for the

client-server pair and shall cause a new subscription to be handled by the Server. If the value is 'update', the server shall update the

previously stored request, i.e., the existing request with the same serial number, according to the new information. The update feature

shall not be used to change the message id of the message field of the Subscription Data structure. If such a change is necessary, the

existing subscription shall be cancelled and a new subscription submitted."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Status"

DATA TYPE ENUMERATED { new, update }

VALUE RULE "ENUMERATED { new, update}"

}

datexSubscribe-Mode			SubscriptionMode,

datexSubscribe-PublishFormat-cd		ENUMERATED { other, ftp, tftp, dataPacket,... },

datexSubscribe-PublishFormat-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_PublishFormat_code-datex4"

CONTEXT "ITS"

DEFINITION "The format in which the corresponding publication(s) should be exchanged. If the value is 'ftp' or 'tftp', the server shall store the

publication (reply) in a file and transmit the file name in the 'publication' data packet. If the value is 'dataPacket' the server

shall attempt to transmit the publication data within the 'publication' data packet."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Publish Format"

DATA TYPE ENUMERATED {other, ftp, tftp, dataPacket, …}

VALUE RULE "ENUMERATED {other, ftp, tftp, dataPacket, …}"

}

datexSubscription-Priority-nbr		INTEGER(1..10),

datexSubscribe-Priority-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_Priority_code-datex11"

CONTEXT "ITS"

DEFINITION "Indicates the relative priority of the subscription. A priority of 1 is the highest priority and a priority of 10 is the lowest priority."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-DATEX Priority"

DATA TYPE INTEGER (1..10)

VALUE RULE "INTEGER (1..10)"

}

datexSubscribe-Guarantee-bool		BOOLEAN,

datexSubscribe-Guarantee-bool DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.SUBSCRIBE_Guarantee_boolean"

CONTEXT "ITS"

DEFINITION "Indicates whether or not the publication (reply) should be confirmed. If the value is 'true', the server shall set

DATEX.PUBLISH_Guaranteed_boolean of the publication data packet(s) to true; otherwise the indicated field shall be set to false."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Boolean"

DATA TYPE BOOLEAN

VALUE RULE "BOOLEAN"

}

datexSubscribe-Pdu					EndApplicationMessage

}

SubscriptionMode ::= CHOICE

{

single		NULL,

event-driven		Registered,

periodic		Registered

}

Registered::= CHOICE

{

continuous		SEQUENCE

{

datexRegistered-UpdateDelay-qty	INTEGER (0..4294967295)  DEFAULT 0,	-- 0 means as soon as possible

datexRegistered-UpdateDelay-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_UpdateDelay_quantity-ulong"

CONTEXT "ITS"

DEFINITION "The update interval for the subscription (request).

If the mode field of the SubscriptionData structure is 'periodic', a new publication (reply) shall be produced every DATEX.REGISTERED_UpdateDelay_quantityulong seconds.

If the mode field of the SubscriptionData structure is 'event-driven', a new publication data packet shall be generated within DATEX.REGISTERED_UpdateDelay_quantity seconds from the time that a new record or record update occurs, respectively, as defined by the message referenced in the message field of the same SubscriptionData structure;

in this case, the value denotes a maximum latency."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

datexRegistered-StartTime	IsoTime	OPTIONAL,			-- defaults to immediate

datexRegistered-StartTime DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_StartTime_frame"

CONTEXT "ITS"

DEFINITION "The time of day at which the subscription (request) is activated, on those days which are valid according to the StartDate, EndDate and DaysOfWeek fields.

If the date on which this subscription is received is a valid day and the start time has already passed but the stop time has not passed, the subscription shall immediately be activated.

The value 00:00 shall mean the start of the day."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

datexRegistered-EndTime	IsoTime	OPTIONAL			-- defaults to "until cancelled"

datexRegistered-EndTime DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_EndTime_frame"

CONTEXT "ITS"

DEFINITION "The time of day at which the subscription (request) is deactivated. "

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

},

daily		SEQUENCE

{

datexRegistered-UpdateDelay-qty	INTEGER (0..4294967295) 	DEFAULT 0,	-- 0 means as soon as possible

datexRegistered-UpdateDelay-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_UpdateDelay_quantity-ulong"

CONTEXT "ITS"

DEFINITION "The update interval for the subscription (request).

If the mode field of the SubscriptionData structure is 'periodic', a new publication (reply) shall be produced every DATEX.REGISTERED_UpdateDelay_quantityulong seconds.

If the mode field of the SubscriptionData structure is 'event-driven', a new publication data packet shall be generated within DATEX.REGISTERED_UpdateDelay_quantity seconds from the time that a new record or record update occurs, respectively, as defined by the message referenced in the message field of the same SubscriptionData structure; in this case, the value denotes a maximum latency."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-ULONG"

DATA TYPE INTEGER (0..4294967295)

VALUE RULE "INTEGER (0..4294967295)"

}

datexRegistered-DaysOfWeek-cd	BIT STRING				-- has to be checked

{

other(0),

sunday(1),

monday(2),

tuesday(3),

wednesday(4),

thursday(5),

friday(6),

saturday(7)

} (SIZE(8)),

datexRegistered-DaysOfWeek-cd DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_DaysOfWeek_code-DaysOfWeek"

CONTEXT "ITS"

DEFINITION "The days of week on which the subscription (request) is activated on the server. "

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Code-Days Of Week"

DATA TYPE BIT STRING {other, sunday, monday, tuesday, wednesday, thursday, friday, saturday} (SIZE (8))

VALUE RULE "BIT STRING {other, sunday, monday, tuesday, wednesday, thursday, friday, saturday} (SIZE (8))"

}

datexRegistered-StartDate	IsoTime		OPTIONAL,		-- defaults to immediate

datexRegistered-StartDate DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_StartDate_frame"

CONTEXT "ITS"

DEFINITION "The first date on which the subscription (request) may be activated."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

datexRegistered-EndDate	IsoTime		OPTIONAL,		-- defaults to "until cancelled"

datexRegistered-EndDate DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_EndDate_frame"

CONTEXT "ITS"

DEFINITION "The last date on which the subscription (request) may be activated."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

datexRegistered-StartTime	IsoTime		OPTIONAL,		-- defaults to midnight

datexRegistered-StartTime DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_StartTime_frame"

CONTEXT "ITS"

DEFINITION "The time of day at which the subscription (request) is activated, on those days which are valid according to the StartDate, EndDate and

DaysOfWeek fields. If the date on which this subscription is received is a valid day and the start time has already passed but the stop time has not passed, the subscription shall immediately be activated. The value 00:00 shall mean the start of the day."

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Time data structure per A.8"

DATA TYPE Time

VALUE RULE "Time data structure per A.8"

}

datexRegistered-Duration-qty	INTEGER (0..65535)	OPTIONAL		-- defaults to 1440 (i.e., 24 hours)

datexRegistered-Duration-qty DATA-ELEMENT ::= {

DESCRIPTIVE-NAME "DATEX.REGISTERED_Duration_quantity-ushort"

CONTEXT "ITS"

DEFINITION "The duration, in minutes, for which the subscription (request) remains active after the Start Time. "

CLASS "Communication Networks"

CLASS SCHEME "ITS Classification Scheme"

SCHEME VERSION "980201"

VALUE DOMAIN "Qty-USHORT"

DATA TYPE INTEGER (0..65535)

VALUE RULE "INTEGER (0..65535)"

}

}

}